VAX (MicroVAX 3900) Simulator Usage

23-Apr-2018

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2016, written by Robert M Supnik

Copyright (c) 1993-2016, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	VAX Features	4

2.1	CPU and System Devices	5

2.1.1	CPU	5

2.1.2	Translation Buffer (TLB)	7

2.1.3	Qbus Adapter (QBA)	7

2.1.4	Read-only memory (ROM)	8

2.1.5	Non-volatile Memory (NVR)	8

2.1.6	System Devices (SYSD)	8

2.2	I/O Device Addressing	9

2.3	Programmed I/O Devices	10

2.3.1	Terminal Input (TTI)	10

2.3.2	Terminal Output (TTO)	10

2.3.3	LPV11 Line Printer (LPT)	11

2.3.4	Real-Time Clock (CLK)	11

2.4	Disks	12

2.4.1	RLV12/RL01,RL02 Cartridge Disk (RL)	12

2.4.2	RQDX3 MSCP Disk Controllers (RQ, RQB, RQC, RQD)	13

2.5	Tapes	14

2.5.1	TSV11/TSV05 Magnetic Tape (TS)	14

2.5.2	TQK50 TMSCP Tape Controller (TQ)	15

2.6	Communications Devices	17

2.6.1	DZV11 Terminal Multiplexer (DZ)	17

2.6.2	DHQ11 Terminal Multiplexer (VH)	18

2.6.3	DELQA-T/DELQA/DEQNA Qbus Ethernet Controllers (XQ, XQB)	20

2.7	CR11 Card Reader (CR)	21

3	Symbolic Display and Input	23

Appendix - The KA655"X"	24

This memorandum documents the DEC VAX (MicroVAX 3900) simulator.

# Simulator Files

To compile the VAX, you must define VM_VAX and USE_INT64 as part of the compilation command line.  To enable extended file support (files greater than 2GB), you must define USE_ADDR64 as part of the command line as well.

sim/		scp.h

sim_console.h

sim_defs.h

sim_disk.h

sim_ether.h

sim_fio.h

sim_rev.h

sim_serial.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_disk.c

sim_ether.c

sim_fio.c

sim_serial.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/vax/	vax_defs.h

vax_ka655x_bin.h

vaxmod_defs.h

vax_cis.c

vax_cmode.c

vax_cpu.c

vax_cpu1.c

vax_fpa.c

vax_io.c

vax_mmu.c

vax_octa.c

vax_stddev.c

vax_sys.c

vax_syscm.c

vax_sysdev.c

vax_syslist.c

sim/pdp11/	pdp11_cr_dat.h

pdp11_mscp.h

pdp11_uqssp.h

pdp11_xq.h

pdp11_xq_bootrom.h

pdp11_cr.c

pdp11_lp.c

pdp11_rl.c

pdp11_rq.c

pdp11_tq.c

pdp11_ts.c

pdp11_vh.c

pdp11_xq.c

Additional files are:

sim/vax/	ka655x.bin		extended memory boot ROM code

# VAX Features

The VAX simulator is configured as follows:

device name(s)		simulates

CPU			KA655”X” CPU with 16MB-512MB of memory

TLB			translation buffer

ROM			read-only memory

NVR			non-volatile memory

QBA			Qbus adapter

SYSD			system devices

TTI,TTO		console terminal

CLK			real-time clock

DZ			DZV11 4-line terminal multiplexer (up to 32)

VH			DHQ11 8-line terminal multiplexer (up to 4)

CR			CR11 card reader

LPT			LPV11 line printer

RL			RLV12/RL01(2) cartridge disk controller with four drives

RQ			RQDX3 MSCP controller with four drives

RQB			second RQDX3 MSCP controller with four drives

RQC			third RQDX3 MSCP controller with four drives

RQD			fourth RQDX3 MSCP controller with four drives

TS			TSV11/TSV05 magnetic tape controller with one drive

TQ			TQK50 TMSCP magnetic tape controller with four drives

XQ			DELQA/DEQNA Ethernet controller

XQB			second DELQA/DEQNA Ethernet controller

The CR, DZ, VH, LPT, RL, RQ, RQB, RQC, RQD, RY, TS, TQ, XQ, and XQB devices can be set DISABLED.  RQB, RQC, RQD, VH, and XQB are disabled by default.

The VAX simulator implements several unique stop conditions:

- Change mode to interrupt stack

- Illegal vector (bits<1:0> = 2 or 3)

- Unexpected exception during interrupt or exception

- Process PTE in P0 or P1 space instead of system space

- Unknown IPL

- Infinite loop (BRB/W to self at IPL 1F)

The LOAD command supports a simple binary format, consisting of a stream of binary bytes without origin or checksum, for loading memory, the boot ROM, or the non-volatile memory.  The DUMP command is not implemented.

## CPU and System Devices

### CPU

CPU options include the size of main memory and the treatment of the HALT instruction.

SET CPU 16M				set memory size = 16MB

SET CPU 32M				set memory size = 32MB

SET CPU 48M				set memory size = 48MB

SET CPU 64M				set memory size = 64MB

SET CPU 128M			set memory size = 128MB

SET CPU 256M			set memory size = 256MB

SET CPU 512M			set memory size = 512MB

SET CPU SIMHALT			kernel HALT returns to simulator

SET CPU CONHALT			kernel HALT returns to boot ROM console

The CPU also implements a command to display a virtual to physical address translation:

SHOW {-kesu} CPU VIRTUAL=n	show translation for address n

in kernel/exec/supervisor/user mode

Notes on memory size:

- The real KA655 CPU only supported 16MB to 64MB of memory.  The simulator implements a KA655"X", which increases supported memory to 512MB.

- The firmware (ka655x.bin) contains code to determine the size of extended memory and set up the PFN bit map accordingly.  Other than setting up the PFN bit map, the firmware does not recognize extended memory and will behave as though memory size was 64MB.

- If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.

- If the simulator is running VMS, the operating system may have a SYSGEN  parameter set called PHYSICAL PAGES (viewable from "MCR SYSGEN SHOW PHYSICALPAGES").  PHYSICALPAGES limits the maximum number of physical pages of memory the OS will recognize.  If it is set to a lower value than the new memory size of the machine, then only the first PHYSICALPAGES of memory will be recognized, otherwise the actual size of the extended memory will be realized by VMS upon each boot.  Some users and/or sites may specify the PHYSICALPAGES parameter in the input file to AUTOGEN (SYS$SYSTEM:MODPARAMS.DAT).  If PHYSICALPAGES is specified there, it will have to be adjusted before running AUTOGEN to recognize more memory.  The default value for PHYSICALPAGES is 1048576, which describes 512MB of RAM.

Initial memory size is 16MB.

Memory can be loaded with a binary byte stream using the LOAD command.  The LOAD command recognizes three switches:

-o			origin argument follows file name

-r			load the boot ROM

-n			load the non-volatile RAM

The CPU supports the BOOT command and is the only VAX device to do so.  Note that the behavior of the bootstrap depends on the capabilities of the console terminal emulator.  If the terminal window supports full VT100 emulation (including Multilanguage Character Set support), the bootstrap will ask the user to specify the language; otherwise, it will default to English.

These switches are recognized when examining or depositing in CPU memory:

-b			examine/deposit bytes

-w			examine/deposit words

-l			examine/deposit longwords

-d			data radix is decimal

-o			data radix is octal

-h			data radix is hexadecimal

-m			examine (only) VAX instructions

-p			examine/deposit PDP-11 (compatibility mode) instructions

-r			examine (only) RADIX50 encoded data

-v			interpret address as virtual, current mode

-k			interpret address as virtual, kernel mode

-e			interpret address as virtual, executive mode

-s			interpret address as virtual, supervisor mode

-u			interpret address as virtual, user mode

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			32		program counter

R0 .. R14		32		R0 to R14

AP			32		alias for R12

FP			32		alias for R13

SP			32		alias for R14

PSL			32		processor status longword

CC			4		condition codes, PSL<3:0>

KSP			32		kernel stack pointer

ESP			32		executive stack pointer

SSP			32		supervisor stack pointer

USP			32		user stack pointer

IS			32		interrupt stack pointer

SCBB			32		system control block base

PCBB			32		process control block base

P0BR			32		P0 base register

P0LR			22		P0 length register

P1BR			32		P1 base register

P1LR			22		P1 length register

SBR			32		system base register

SLR			22		system length register

SISR			16		software interrupt summary register

ASTLVL		4		AST level register

MAPEN			1		memory management enable

PME			1		performance monitor enable

TRPIRQ		8		trap/interrupt pending

CRDERR		1		correctible read data error flag

MEMERR		1		memory error flag

PCQ[0:63]		32		PC prior to last PC change or interrupt;

most recent PC change first

WRU			8		interrupt character

The CPU attempts to detect when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE{=VMS|ULTRIX| ULTRIXOLD|ULTRIX-1.X|

3BSD|4.0BSD|4.1BSD|4.2BSD|QUASIJARUS

NETBSD|NETBSDOLD|OPENBSD|OPENBSDOLD|32V|ELN}{:n}

enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default.  If idle detection is enabled with an incorrect operating system setting, simulator performance could be impacted.  The default operating system setting is VMS.  The value ‘n’, if present in the “SET CPU IDLE={OS}:n” command, indicates the number of seconds the simulator must run before idling starts.

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU {-T} HISTORY=n{:file}	enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The –T switch causes simulator time to be recorded (and displayed) with each history entry.

When writing history to a file (SET CPU HISTORY=n:file), ‘n’ specifies the buffer flush frequency.  Warning: prodigious amounts of disk space may be consumed.  The maximum length for the history is 250000 entries.

### Translation Buffer (TLB)

The translation buffer consists of two units, representing the system and user translation buffers, respectively.  It has no registers.  Each translation buffer entry consists of two 32b words, as follows:

word n		tag

word n+1		cached PTE

An invalid entry is indicated by a tag of 0xFFFFFFFF.

### Qbus Adapter (QBA)

The QBA simulates the CQBIC Qbus adapter chip.  It recognizes the following options:

SET QBA AUTOCONFIGURE		enable autoconfiguration

SET QBA NOAUTOCONFIGURE		disable autoconfiguration

and the following display command:

SHOW QBA IOSPACE			show I/O space address map

The QBA also implements a command to display a Qbus address to physical address translation:

SHOW QBA VIRTUAL=n		show translation for Qbus address n

Finally, the QBA implements main memory examination and modification via the Qbus map.  The data width is always 16b:

EX QBA 0/10				examine main memory words corresponding

to Qbus addresses 0-10

The QBA registers are:

name			size		comments

SCR			16		system configuration register

DSER			8		DMA system error register

MEAR			13		master error address register

SEAR			20		slave error address register

MBR			29		Qbus map base register

IPC			16		interprocessor communications register

IPL17			32		IPL 17 interrupt flags

IPL16			32		IPL 16 interrupt flags

IPL15			32		IPL 15 interrupt flags

IPL14			32		IPL 14 interrupt flags

### Read-only memory (ROM)

The boot ROM consists of a single unit, simulating the 128KB boot ROM.  It has no registers.  The boot ROM is loaded with a binary byte stream using the LOAD -r command:

LOAD -r KA655X.BIN		load ROM image KA655X.BIN

ROM accesses a use a calibrated delay that slows ROM-based execution to about 500K instructions per second.  This delay is required to make the power-up self-test routines run correctly on very fast hosts.  The delay is controlled with the commands:

SET ROM NODELAY			ROM runs like RAM

SET ROM DELAY			ROM runs slowly

### Non-volatile Memory (NVR)

The NVR consists of a single unit, simulating 1KB of battery-backed up memory in the SSC chip.  When the simulator starts, NVR is cleared to 0, and the SSC battery-low indicator is set.  Normally, NVR is saved and restored like other memory in the system.  Alternately, NVR can be attached to a file.  This allows its contents to be saved and restored independently of other memories, so that NVR state can be preserved across simulator runs.

Successfully loading an NVR image clears the SSC battery-low indicator.

### System Devices (SYSD)

The system devices are the system-specific facilities implemented in the CVAX chip, the KA655 CPU board, the CMCTL memory controller, and the SSC system support chip.  Note that the simulation of these devices is incomplete and is intended strictly to allow the patched bootstrap and console code to run.  The SYSD registers are:

name			size		comments

CADR			8		cache disable register

MSER			8		memory system error register

CONPC			32		PC at console halt

CONPSL		32		PSL at console halt

CMCSR[0:17]		32		CMCTL control and status registers

CACR			8		second-level cache control register

BDR			8		front panel jumper register

BASE			29		SSC base address register

CNF			32		SSC configuration register

BTO			32		SSC bus timeout register

TCSR0			32		SSC timer 0 control/status register

TIR0			32		SSC timer 0 interval register

TNIR0			32		SSC timer 0 next interval register

TIVEC0		9		SSC timer 0 interrupt vector register

TCSR1			32		SSC timer 1 control/status register

TIR1			32		SSC timer 1 interval register

TNIR1			32		SSC timer 1 next interval register

TIVEC1		9		SSC timer 1 interrupt vector register

ADSM0			32		SSC address match 0 address

ADSK0			32		SSC address match 0 mask

ADSM1			32		SSC address match 1 address

ADSK1			32		SSC address match 1 mask

CDGDAT[0:16383]	32		cache diagnostic data store

BDR<7> is the halt-enabled switch.  It controls how the console firmware responds to a BOOT command, a kernel halt (if option CONHALT is set), or a console halt (BREAK typed on the console terminal).  If BDR<7> is set, the console firmware responds to all these conditions by entering its interactive command mode.  If BDR<7> is clear, the console firmware boots the operating system in response to these conditions.  This bit can be set and cleared by the command “SET CPU AUTOBOOT” (clearing the flag) and “SET CPU NOAUTOBOOT” setting the flag.  The default value is set.

## I/O Device Addressing

Qbus I/O space is not large enough to allow all possible devices to be configured simultaneously at fixed addresses.  Instead, many devices have floating addresses; that is, the assigned device address depends on the presence of other devices in the configuration:

DZ11			all instances have floating addresses

DHQ11			all instances have floating addresses

RL11			first instance has fixed address, rest floating

RXV211		first instance has fixed address, rest floating

MSCP disk		first instance has fixed address, rest floating

TMSCP tape		first instance has fixed address, rest floating

To maintain addressing consistency as the configuration changes, the simulator implements DEC's standard I/O address and vector autoconfiguration algorithms for all Qbus devices.  This allows the user to enable or disable devices without needing to manage I/O addresses and vectors.

In addition to autoconfiguration, most devices support the SET <device> ADDRESS command, which allows the I/O page address of the device to be changed, and the SET <device> VECTOR command, which allows the vector of the device to be changed.  Explicitly setting the I/O address or vector of a device DISABLES autoconfiguration for that device and for the entire system.  As a consequence, the user may have to manually configure all other autoconfigured devices, because the autoconfiguration algorithm no longer recognizes the explicitly configured device.  A device can be reset to autoconfigure with the SET <device> AUTOCONFIGURE command.  Autoconfiguration can be restored for the entire system with the SET QBA AUTOCONFIGURE command.

The current I/O map can be displayed with the SHOW QBA IOSPACE command.  Addresses that have set by autoconfiguration are marked with an asterisk (*).

All devices support the SHOW <device> ADDRESS and SHOW <device> VECTOR commands, which display the device address and vector, respectively.

## Programmed I/O Devices

### Terminal Input (TTI)

The terminal interfaces (TTI, TTO) can be set to one of three modes, 7P, 7B or 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is 8B.

When the console terminal is attached to a Telnet session or the simulator is running from a Windows command prompt, it recognizes BREAK.  If BREAK is entered, and BDR<7> is set, control returns to the console firmware; otherwise, BREAK is treated as a normal terminal input condition.

The terminal input (TTI) polls the console keyboard for input.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters input

TIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the TODR)

### Terminal Output (TTO)

The terminal output (TTO) writes to the simulator console window.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters input

TIME			24		time from I/O initiation to interrupt

### LPV11 Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

### Real-Time Clock (CLK)

The clock (CLK) implements these registers:

name			size		comments

CSR			16		control/status register

INT			1		interrupt pending flag

IE			1		interrupt enable flag (CSR<6>)

TODR			32		time-of-day register

BLOW			1		TODR battery low indicator

TIME			24		clock frequency

TPS			8		ticks per second (100)

The real-time clock autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.

There are two modes of TODR operation.

- Default VMS mode.  Without initializing the TODR it returns the current time of year offset which VMS would set the clock to if VMS knew the correct time (i.e. by manual input).  This is correct almost all the time unless a VMS disk hadn’t been booted from in the current year.  This mode produces strange time results for non VMS OSes on each system boot.

- OS Agnostic mode.  This mode behaves precisely like the VAX780 TODR and works correctly for all OSes.  This mode is enabled by attaching the CLK to a battery backup state file for the TOY clock (i.e. sim> attach CLK TOY_CLOCK).  When operating in OS Agnostic mode, the TODR will initially start counting from 0 and be adjusted differently when an OS specifically writes to the TODR.  VMS determines if the TODR currently contains a valid time if the value it sees is less than about 1 month.  If the time isn't valid VMS will prompt to set the time during the system boot.  While prompting for the time it will wait for an answer to the prompt for up to the SYSGEN parameter TIMEPROMPTWAIT seconds.  A value of 0 for TIMEPROMPTWAIT will disable the clock setting prompt.

## Disks

### RLV12/RL01,RL02 Cartridge Disk (RL)

RLV12 options include the ability to set units write enabled or write locked, to set the drive type to RL01, RL02, or autosize, and to write a DEC standard 044 compliant bad block table on the last track:

SET RLn LOCKED			set unit n write locked

SET RLn WRITEENABLED		set unit n write enabled

SET RLn RL01			set type to RL01

SET RLn RL02			set type to RL02

SET RLn AUTOSIZE			set type based on file size at ATTACH

SET RLn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can also be set ENABLED or DISABLED.  The RLV12 does not support the BOOT command.

The RLV12 implements these registers:

name			size		comments

RLCS			16		control/status

RLDA			16		disk address

RLBA			16		memory address

RLBAE			6		memory address extension (RLV12)

RLMP,RLMP1,RLMP2	16		multipurpose register queue

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RQDX3 MSCP Disk Controllers (RQ, RQB, RQC, RQD)

The simulator implements four MSCP disk controllers, RQ, RQB, RQC, RQD.  Initially, RQB, RQC, and RQD are disabled.  Each RQ controller simulates an RQDX3 MSCP disk controller with four drives.  RQ options include the ability to set units write enabled or write locked, and to set the drive type to one of many disk types:

SET RQn LOCKED			set unit n write locked

SET RQn WRITEENABLED		set unit n write enabled

SET RQn RX50			set type to RX50

SET RQn RX33			set type to RX33

SET RQn RD32			set type to RD32

SET RQn RD51			set type to RD51

SET RQn RD52			set type to RD52

SET RQn RD53			set type to RD53

SET RQn RD54			set type to RD54

SET RQn RD31			set type to RD31

SET RQn RA80			set type to RA80

SET RQn RA81			set type to RA81

SET RQn RA82			set type to RA82

set RQn RA71			set type to RA71

SET RQn RA72			set type to RA72

SET RQn RA90			set type to RA90

SET RQn RA92			set type to RA92

SET RQn RRD40			set type to RRD40 (CD ROM)

SET RQn RAUSER{=n}		set type to RA82 with n MB's

SET -L RQn RAUSER{=n}		set type to RA82 with n LBN's

The type options can be used only when a unit is not attached to a file.  RAUSER is a "user specified" disk; the user can specify the size of the disk in either MB (1000000 bytes) or logical block numbers (LBN's, 512 bytes each).  The minimum size is 5MB; the maximum size is 2GB without extended file support, 1TB with extended file support

Units can also be set ENABLED or DISABLED.

Drive units have changeable unit numbers.  Unit numbers can be changed with:

SET RQn UNIT=val			set unit plug value

Each device has 4 units which have unique MSCP unit numbers (0, 1, 2 and 3).

Each RQ controller implements the following special SHOW commands:

SHOW RQn TYPE			show drive type

SHOW RQ RINGS			show command and response rings

SHOW RQ FREEQ			show packet free queue

SHOW RQ RESPQ			show packet response queue

SHOW RQ UNITQ			show unit queues

SHOW RQ ALL				show all ring and queue state

SHOW RQn UNITQ			show unit queues for unit n

SHOW RQn UNIT			show unit plug value

Each RQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

PLUG[0:3]		16		unit plug values, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

While VMS is not timing sensitive, most of the BSD-derived operating systems (NetBSD, OpenBSD, etc) are.  The QTIME and XTIME parameters are set to values that allow these operating systems to run correctly.

Error handling is as follows:

error	     		processed as

not attached	disk not ready

end of file		assume rest of disk is zero

OS I/O error	report error and stop

## Tapes

### TSV11/TSV05 Magnetic Tape (TS)

TS options include the ability to make the unit write enabled or write locked.

SET TS LOCKED			set unit write locked

SET TS WRITEENABLED		set unit write enabled

The TS drive can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TS0 CAPAC=m			set capacity to m MB (0 = unlimited)

SHOW TS0 CAPAC			show capacity in MB

The TSV11 does not support the BOOT command.

The TS controller implements these registers:

name			size		comments

TSSR			16		status register

TSBA			16		bus address register

TSDBX			16		data buffer extension register

CHDR			16		command packet header

CADL			16		command packet low address or count

CADH			16		command packet high address

CLNT			16		command packet length

MHDR			16		message packet header

MRFC			16		message packet residual frame count

MXS0			16		message packet extended status 0

MXS1			16		message packet extended status 1

MXS2			16		message packet extended status 2

MXS3			16		message packet extended status 3

MXS4			16		message packet extended status 4

WADL			16		write char packet low address

WADH			16		write char packet high address

WLNT			16		write char packet length

WOPT			16		write char packet options

WXOPT			16		write char packet extended options

ATTN			1		attention message pending

BOOT			1		boot request pending

OWNC			1		if set, tape owns command buffer

OWNM			1		if set, tape owns message buffer

TIME			24		delay

POS			32		position

Error handling is as follows:

error			processed as

not attached	tape not ready

end of file		bad tape

OS I/O error	fatal tape error

### TQK50 TMSCP Tape Controller (TQ)

The TQ controller simulates the TQK50 TMSCP tape controller.  TQ options include the ability to set units write enabled or write locked, and to specify the controller type and tape length:

SET TQn LOCKED			set unit n write locked

SET TQn WRITEENABLED		set unit n write enabled

SET TQ TK50				set controller type to TK50

SET TQ TK70				set controller type to TK70

SET TQ TU81				set controller type to TU81

SET TQ TKUSER{=n}			set controller type to TK50 with

tape capacity of n MB

User-specified capacity must be between 50 and 2000000000 MB.

Regardless of the controller type, individual units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TQn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TQn CAPAC			show unit n capacity in MB

Drive units have changeable unit numbers.  Unit numbers can be changed with:

SET TQn UNIT=val			set unit plug value

Device TQ has 4 units (TQ0, TQ1, TQ2 and TQ3) which have unique MSCP unit numbers (0, 1, 2 and 3).

The TQ controller implements the following special SHOW commands:

SHOW TQ TYPE			show controller type

SHOW TQ RINGS			show command and response rings

SHOW TQ FREEQ			show packet free queue

SHOW TQ RESPQ			show packet response queue

SHOW TQ UNITQ			show unit queues

SHOW TQ ALL				show all ring and queue state

SHOW TQn UNITQ			show unit queues for unit n

SHOW TQn UNIT			show unit plug value

The TQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

PLUG[0:3]		16		unit plug values, units 0 to 3

POS[0:3]		32		tape position, units 0 to 3

OBJP[0:3]		32		object position, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

Error handling is as follows:

error	     		processed as

not attached	tape not ready

end of file		end of medium

OS I/O error	fatal tape error

## Communications Devices

### DZV11 Terminal Multiplexer (DZ)

The DZV11 is a 4-line terminal multiplexor.  Up to 4 DZ11's (16 lines) are supported.  The number of lines can be changed with the command

SET DZ LINES=n			set line count to n

The line count must be a multiple of 4, with a maximum of 16.

The DZ11 supports three character processing modes, 7P, 7B, and 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default is 8B.

The DZV11 supports logging on a per-line basis.  The command

SET DZ LOG=line=filename

enables logging for the specified line to the indicated file.  The command

SET DZ NOLOG=line

disables logging for the specified line and closes any open log file.  Finally, the command

SHOW DZ LOG

displays logging information for all DZ lines.

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH {-am} DZ <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The optional switch -m turns on the DZV11's modem controls; the optional switch -a turns on active disconnects (disconnect session if computer clears Data Terminal Ready).  Without modem control, the DZV11 behaves as though terminals were directly connected; disconnecting the Telnet session does not cause any operating system-visible change in line status.

Once the DZ is attached and the simulator is running, the DZ will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET DZ DISCONNECT command, or a DETACH DZ command.

Other special DZ commands:

SHOW DZ CONNECTIONS 			show current connections

SHOW DZ STATISTICS 			show statistics for active connections

SET DZ DISCONNECT=linenumber		disconnects the specified line.

The DZV11 implements these registers:

name			size		comments

CSR[0:3]		16		control/status register, boards 0 to 3

RBUF[0:3]		16		receive buffer, boards 0 to 3

LPR[0:3]		16		line parameter register, boards 0 to 3

TCR[0:3]		16		transmission control register, boards 0 to 3

MSR[0:3]		16		modem status register, boards 0 to 3

TDR[0:3]		16		transmit data register, boards 0 to 3

SAENB[0:3]		1		silo alarm enabled, boards 0 to 3

RXINT			4		receive interrupts, boards 3 to 0

TXINT			4		transmit interrupts, boards 3 to 0

MDMTCL		1		modem control enabled

AUTODS		1		autodisconnect enabled

The DZV11 partially supports save and restore.  A save simulator state will restore the listening sockets and serial port parameters, but all active incoming telnet connections will be lost.

All active incoming telnet connections are lost when the simulator shuts down or the DZ is detached.

### DHQ11 Terminal Multiplexer (VH)

The DHQ11 is an 8-line terminal multiplexer for Qbus systems.  Up to 4 DHQ11's are supported.

The DHQ11 is a programmable asynchronous terminal multiplexer.  It has two programming modes: DHV11 and DHU11.  The register sets are compatible with these devices.  For transmission, the DHQ11 can be used in either DMA or programmed I/O mode.  For reception, there is a 256-entry FIFO for received characters, dataset status changes, and diagnostic information, and a programmable input interrupt timer (in DHU mode).  The device supports 16-, 18-, and 22-bit addressing.  The DHQ11 can be programmed to filter and/or handle XON/XOFF characters independently of the processor.  The DHQ11 supports programmable bit width (between 5 and 8) for the input and output of characters.

The DHQ11 has a rocker switch for determining the programming mode.  By default, the DHV11 mode is selected, though DHU11 mode is recommended for applications that can support it.  The VH controller may be adjusted on a per controller basis as follows:

SET VHn DHU				use the DHU programming mode and registers

SET VHn DHV				use the DHV programming mode and registers

DMA output is supported.  In a real DHQ11, DMA is not initiated immediately upon receipt of TX.DMA.START but is dependent upon some internal processes.  The VH controller mimics this behavior by default.  It may be desirable to alter this and start immediately, though this may not be compatible with all operating systems and diagnostics.  You can change the behavior of the VH controller as follows:

SET VHn NORMAL			use normal DMA procedures

SET VHn FASTDMA			set DMA to initiate immediately

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH VH <port>			set up listening port

where port is a decimal number between 1 and 65535 that is not  being used for other TCP/IP activities.  This port is the point of entry for all lines on all VH controllers.

The number of lines can be changed with the command

SET VH LINES=n			set line count to n

The line count must be a multiple of 8, with a maximum of 32.

Modem and auto-disconnect support may be set on an individual controller basis.  The SET MODEM command directs the controller to report modem status changes to the computer.  The SET HANGUP command turns on active disconnects (disconnect session if computer clears Data Terminal Ready).

SET VHn [NO]MODEM			disable/enable modem control

SET VHn [NO]HANGUP		disable/enable disconnect on DTR drop

Once the VH is attached and the simulator is running, the VH will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET VH DISCONNECT command, or a DETACH VH command.

Other special VH commands:

SHOW VH CONNECTIONS 		show current connections

SHOW VH STATISTICS 		show statistics for active connections

SET VH DISCONNECT=linenumber	disconnects the specified line.

The DHQ11 implements these registers, though not all can be examined from SCP:

name            size    	comments

CSR[0:3]        16      	control/status register, boards 0 to 3

RBUF[0:3]       16      	receive buffer, boards 0 to 3

LPR[0:3]        16      	line parameter register, boards 0 to 3

RXINT           4       	receive interrupts, boards 3..0

TXINT           4       	transmit interrupts, boards 3..0

[more to be described...]

The DHQ11 does not support save and restore.  All open connections are lost when the simulator shuts down or the VH is detached.

### DELQA-T/DELQA/DEQNA Qbus Ethernet Controllers (XQ, XQB)

The simulator implements two DELQA-T/DELQA/DEQNA Qbus Ethernet controllers (XQ, XQB).  Initially, XQ is enabled, and XQB is disabled.  Options allow control of the MAC address, the controller mode, and the sanity timer.

SET  XQ MAC=<mac-address>	ex. 08-00-2B-AA-BB-CC

SHOW XQ MAC

These commands are used to change or display the MAC address.  <mac-address> is a valid Ethernet MAC, delimited by dashes or periods.  The controller defaults to 08-00-2B-AA-BB-CC, which should be sufficient if there is only one SIMH controller on your LAN.  Two cards with the same MAC address will see each other's packets, resulting in a serious mess.

SET  XQ TYPE={DEQNA|[DELQA]|DELQA-T}

SHOW XQ TYPE

These commands are used to change or display the controller mode. DELQA mode is better and faster but may not be usable by older or non-DEC OS's.  Also, be aware that DEQNA mode is not supported by many modern OS's. The DEQNA-LOCK mode of the DELQA card is emulated by setting the the controller to DEQNA -- there is no need for a separate mode.  DEQNA-LOCK mode behaves exactly like a DEQNA, except for the operation of the VAR and MOP processing.

SET  XQ SANITY={ON|[OFF]}

SHOW XQ SANITY

These commands change or display the INITIALIZATION sanity timer (DEQNA jumper W3/DELQA switch S4).  The INITIALIZATION sanity timer has a default timeout of 4 minutes, and cannot be turned off, just reset.  The normal sanity timer can be set by operating system software regardless of the state of this switch.  Note that only the DEQNA (or the DELQA in DEQNA-LOCK mode (=DEQNA)) supports the sanity timer -- it is ignored by a DELQA in Normal mode, which uses switch S4 for a different purpose.

SET  XQ POLL={DEFAULT|4..2500}

SHOW XQ POLL

These commands change or display the service polling timer.  The polling timer is calibrated to run the service thread 200 times per second.  This value can be changed to accommodate particular system requirements for more (or less) frequent polling.

SHOW XQ STATS

This command will display the accumulated statistics for the simulated Ethernet controller.

To access the network, the simulated Ethernet controller must be attached

to a real Ethernet interface:

ATTACH XQ0 {ethX|<device_name>}	ex. eth0 or /dev/era0

SHOW XQ ETH

where X in 'ethX' is the number of the Ethernet controller to attach, or the real device name.  The X number is system-dependant.  If you only have one Ethernet controller, the number will probably be 0. To find out what your system thinks the Ethernet numbers are, use the SHOW XQ ETH command.  The device list can be quite cryptic, depending on the host system, but is probably better than guessing. If you do not attach the device, the controller will behave as though the Ethernet cable were unplugged.

XQ and XQB have the following registers:

name			size		comments

SA0			16		station address word 0

SA1			16		station address word 1

SA2			16		station address word 2

SA3			16		station address word 3

SA4			16		station address word 4

SA5			16		station address word 5

RBDL			32		receive buffer descriptor list

XBDL			32		trans(X)mit buffer descriptor list

CSR			16		control status register

VAR			16		vector address register

INT			1		interrupt request flag

One final note: because of its asynchronous nature, the XQ controller is not limited to the ~1.5Mbit/sec of the real DEQNA/DELQA controllers, nor the 10Mbit/sec of a standard Ethernet.  Attach it to a Fast Ethernet (100 Mbit/sec) card, and "Feel the Power!" :-)

## CR11 Card Reader (CR)

The card reader (CR) implements a single controller (the CR11) and card reader (e.g., Documation M200, GDI Model 100) by reading a file and presenting lines or cards to the simulator.  Card decks may be represented by plain text ASCII files, card image files, or column binary files.  The CR11 controller is also compatible with the CM11-F, CME11, and CMS11.

Card image files are a file format designed by Douglas W. Jones at the University of Iowa to support the interchange of card deck data.  These files have a much richer information carrying capacity than plain ASCII files.  Card Image files can contain such interchange information as card-stock color, corner cuts, special artwork, as well as the binary punch data representing all 12 columns.  Complete details on the format, as well as sample code, are available at Prof. Jones's site: http://www.cs.uiowa.edu/~jones/cards/.

Examples of the CR11 include the M8290 and M8291 (CMS11).  All card readers use a common vector at 0230 and CSR at 177160.  Even though the CR11 is normally configured as a BR6 device, it is configured for BR4 in this simulation.

The card reader supports ASCII, card image, and column binary format card “decks.”  When reading plain ASCII files, lines longer than 80 characters are silently truncated.  Card image support is included for 80 column Hollerith, 82 column Hollerith (silently ignoring columns 0 and 81), and 40 column Hollerith (mark-sense) cards.  Column binary supports 80 column card images only.  All files are attached read-only (as if the -R switch were given).

ATTACH –A CR <file>			file is ASCII text

ATTACH –B CR <file>			file is column binary

ATTACH –I CR <file>			file is card image format

If no flags are given, the file extension is evaluated.  If the filename ends in .TXT, the file is treated as ASCII text.  If the filename ends in .CBN, the file is treated as column binary.  Otherwise, the CR driver looks for a card image header.  If a correct header is found the file is treated as card image format, otherwise it is treated as ASCII text.

The correct character translation MUST be set if a plain text file is to be used for card deck input.  The correct translation SHOULD be set to allow correct ASCII debugging of a card image or column binary input deck.  Depending upon the operating system in use, how it was generated, and how the card data will be read and used, the translation must be set correctly so that the proper character set is used by the driver.  Use the following command to explicitly set the correct translation:

SET TRANSLATION={DEFAULT|026|026FTN|029|EBCDIC}

This command should be given after a deck is attached to the simulator.  The mappings above are completely described at http://www.cs.uiowa.edu/~jones/cards/codes.html.  Note that DEC typically used 029 or 026FTN mappings.

DEC operating systems used a variety of methods to determine the end of a deck (recognizing that 'hopper empty' does not necessarily mean the end of a deck.  Below is a summary of the various operating system conventions for signaling end of deck:

RT-11:	12-11-0-1-6-7-8-9 punch in column 1

RSTS/E:	12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

RSX:		12-11-0-1-6-7-8-9 punch

VMS:		12-11-0-1-6-7-8-9 punch in first 8 columns

TOPS:		12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

Using the AUTOEOF setting, the card reader can be set to automatically generate an EOF card consisting of the 12-11-0-1-6-7-8-9 punch in columns 1-8.  When set to CD11 mode, this switch also enables automatic setting of the EOF bit in the controller after the EOF card has been processed.  [The CR11 does not have a similar capability.]  By default AUTOEOF is enabled.

SET CR AUTOEOF

SET CR NOAUTOEOF

The default card reader rate for the CR11 is 285 cpm.  The reader rate can be set to its default value or to anywhere in the range 200..1200 cpm.  This rate may be changed while the unit is attached.

SET CR RATE={DEFAULT|200..1200}

It is standard operating procedure for operators to load a card deck and press the momentary action RESET button to clear any error conditions and alert the processor that a deck is available to read.  Use the following command to simulate pressing the card reader RESET button,

SET CR RESET

Another common control of physical card readers is the STOP button.  An operator could use this button to finish the read operation for the current card and terminate reading a deck early.  Use the following command to simulate pressing the card reader STOP button.

SET CR STOP

The simulator does not support the BOOT command.  The simulator does not stop on file I/O errors.  Instead the controller signals a reader check to the CPU.

The CR controller implements these registers:

name			size		comments

BUF			8		ASCII value of last column processed

CRS			16		CR11 status register

CRB1			16		CR11 12-bit Hollerith character

CRB2			16		CR11 8-bit compressed character

CRM			16		CR11 maintenance register

CDST			16		CD11 control/status register

CDCC			16		CD11 column count

CDBA			16		CD11 current bus address

CDDB			16		CD11 data buffer, 2nd status

BLOWER		2		blower state value

INT			1		interrupt pending flag

ERR			1		error flag (CRS<15>)

IE			1		interrupt enable flag (CRS<6>)

POS			32		file position - do not alter

TIME			24		delay time between columns

# Symbolic Display and Input

The VAX simulator implements symbolic display and input.  Display is controlled by command line switches:

-a,-c			display as ASCII data

-m			display instruction mnemonics

-p			display compatibility mode mnemonics

-r			display RADIX50 encoding

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII characters (determined by length)

" or -c		ASCII string (maximum 60 characters)

-p			compatibility mode instruction mnemonic

alphabetic		instruction mnemonic

numeric		octal number

VAX instruction input uses standard VAX assembler syntax.  Compatibility mode instruction input uses standard PDP-11 assembler syntax.

The syntax for VAX specifiers is as follows:

syntax		specifier	displacement	comments

#s^n, #n		0n		-		short literal, integer only

[Rn]			4n		-		indexed, second specifier follows

Rn			5n		-		PC illegal

(Rn)			6n		-		PC illegal

-(Rn)			7n          -		PC illegal

(Rn)+			8n		-

#i^n, #n		8F		n		immediate

@(Rn)+		9n		-

@#addr		9F		addr		absolute

{+/-}b^d(Rn)	An		{+/-}d	byte displacement

b^d			AF		d - PC	byte PC relative

@{+/-}b^d(Rn)	Bn		{+/-}d	byte displacement deferred

@b^d			BF		d - PC	byte PC relative deferred

{+/-}w^d(Rn)	Cn		{+/-}d	word displacement

w^d			CF		d - PC	word PC relative

@{+/-}w^d(Rn)	Dn		{+/-}d	word displacement deferred

@w^d			DF		d - PC	word PC relative deferred

{+/-}l^d(Rn)	En		{+/-}d	long displacement

l^d			EF		d - PC	long PC relative

@{+/-}l^d(Rn)	Fn		{+/-}d	long displacement deferred

@l^d			FF		d - PC	long PC relative deferred

If no override is given for a literal (s^ or i^) or for a displacement or PC relative address (b^, w^, or l^), the simulator chooses the mode automatically.

# Appendix - The KA655"X"

The real KA655 is limited to 64MB of memory, and the KA655 firmware is coded to this limit.  However, the VAX operating systems (VMS, Ultrix, NetBSD) know very little about the hardware details.  Instead, they take their memory size information from the Restart Parameter Block (RPB).  If the firmware sets up an RPB for more than 64MB, the operating systems use the extra memory without requiring source changes.

If more than 64MB of memory is configured, the simulator implements an 18th CMCTL register.  This read-only register gives the size of main memory in MB.  The console firmware (ka655x.bin) uses this to set up the RPB. Other parts of the firmware are generally unaware of extended memory; thus, all the diagnostic commands operate only on the first 64MB of memory.  However SHOW MEM will display the total amount of memory the simulator is configured with

If 64MB or less of memory is configured, the 18th CMCTL register is invisible, and the simulator operates like a real KA655.